📘 Jetson Nano + ESP32

Getting Started Guide (Python 3.10 venv + Serial Control)

1️⃣ Overview

This document explains how to:

- Connect Jetson Nano with ESP32

- Use Python 3.10 virtual environment

- Send ON / OFF commands from Jetson

- Control ESP32 built-in LED

- Troubleshoot common errors

This setup is a foundation for:

- Motor control

- Robotics

- YOLO + actuator integration

- Autonomous rover projects

2️⃣ System Architecture

Jetson Nano (Python 3.10 + YOLO)

│ USB Serial

▼

ESP32 (Arduino code)

│

Built-in LED / Motor Driver

- Jetson Nano = Logic / AI

- ESP32 = Hardware control

- Communication = USB Serial

3️⃣ Requirements

🔧 Hardware

- Jetson Nano (Ubuntu) – from NVIDIA

- ESP32 Dev Board – from Espressif

- USB cable (data cable)

💻 Software

- Arduino IDE (on Laptop / PC)

- ESP32 board support installed

- Python 3.10

- Python virtual environment

- pyserial library

4️⃣ Step 1: Upload ESP32 Code (Laptop / PC)

⚠️ Important
ESP32 code must be uploaded from a laptop/PC (one-time step).

ESP32 Arduino Code (LED Control)

#define LED_PIN 2   // Most ESP32 boards

void setup() {

pinMode(LED_PIN, OUTPUT);

Serial.begin(115200);

}

void loop() {

if (Serial.available()) {

String cmd = Serial.readStringUntil('\n');

cmd.trim();

if (cmd == "ON") {

digitalWrite(LED_PIN, HIGH);

}

else if (cmd == "OFF") {

digitalWrite(LED_PIN, LOW);

}

}

}

Upload Steps

- Open Arduino IDE

- Select Board → ESP32 Dev Module

- Select correct COM / Port

- Click Upload

- If needed, hold BOOT button

✅ “Done uploading” = success

5️⃣ Step 2: Connect ESP32 to Jetson Nano

- Disconnect ESP32 from laptop

- Connect ESP32 to Jetson Nano using USB

- ESP32 will appear as a serial device

Check:

ls /dev/ttyUSB*

Usually:

/dev/ttyUSB0

6️⃣ Step 3: Prepare Python 3.10 Virtual Environment

Activate your venv:

source ~/yolo_env/bin/activate

Install pyserial inside the venv:

pip install pyserial

Verify:

python -c "import serial; print(serial.__version__)"

7️⃣ Step 4: Python Code on Jetson Nano

Create esp32_led_test.py

import serial

import time

PORT = "/dev/ttyUSB0"

BAUD = 115200

ser = serial.Serial(PORT, BAUD, timeout=1)

time.sleep(2)   # ESP32 reset delay

print("LED ON")

ser.write(b"ON\n")

time.sleep(2)

print("LED OFF")

ser.write(b"OFF\n")

time.sleep(2)

ser.close()

Run:

python esp32_led_test.py

8️⃣ Expected Output

On Terminal

LED ON

LED OFF

On ESP32

- Built-in LED turns ON

- Then turns OFF

✅ This confirms:

- USB serial working

- Python venv working

- ESP32 command parsing working

9️⃣ Common Troubleshooting

❌ Error: permission denied /dev/ttyUSB0

✅ Fix (Permanent)

sudo usermod -a -G dialout $USER

reboot

After reboot:

groups

Ensure dialout is listed.

❌ Error: python-c command not found

✔ Wrong command
✔ Correct command:

python -c "import serial"

❌ Error: ModuleNotFoundError: No module named 'serial'

✔ Install inside venv:

pip install pyserial

❌ LED not turning ON

Possible reasons:

- ESP32 LED is active-LOW

- Try swapping:

HIGH ↔ LOW

❌ /dev/ttyUSB0 not found

Check:

ls /dev/tty*

If ESP32 shows as /dev/ttyACM0, update Python code.

❌ ESP32 resets repeatedly

✔ Add delay in Python:

time.sleep(2)

🔟 How This Scales to Motors

This LED test is exactly the same logic for motors:

| LED Test | Motor Control |
| --- | --- |
| ON | Forward |
| OFF | Stop |
| GPIO2 | Motor driver pin |
| LED | DC motor |

Later:

- Replace LED code with motor driver pins

- Send F, S, R, SPD:150

✅ Final Summary

✔ ESP32 flashed once from laptop
✔ Jetson Nano controls via Python
✔ Python 3.10 venv supported
✔ Serial communication verified
✔ Ready for motor & robotics work